SmartMan Manual – 1. Start 0. Install Software
V5.1 and later

 (Updated 20 October  2011)

	Installing SmartMan


The Software
This document provides instructions on how to download and install SmartMan version 5.1+ or later, plus how to install the correct USB driver. 
This SmartMan software is specifically designed for Guidelines 2010 and runs on SmartMan manikins purchased after 1 April 2011 or which have been upgraded to the new guidelines after 1 April 2011. 
Summary of Procedures
Step 1: Download and install software

Step 2: Manually install the USB Driver

Step 3: Start SmartMan

	STEP 1: Download and Unzip the Software


Download: There is one version of the software regardless for SmartMan or Airway SmartMan. Download the software by clicking the link:
http://www.ambusmartman.com/s-update.html
Unzip: Double click on the file that you download. Follow the prompts on this self extracting file that will create the proper directories, install the SmartMan Software and create a shortcut on your desktop.

	STEP 2: Install the USB Driver


If this is a first time installation of v5.1 SmartMan, you must install the USB driver. 

· Plug in the USB cable and choose the option "Browse my computer for Driver software"
Do NOT allow windows to install a driver automatically. 
Point to the driver files which will be located here:
C:\Program Files\SmartManv5.1\SmartManDriver

OR

C:\Program Files (x86)\SmartManv5.1\SmartManDriver

· Detailed driver installation help for different systems is below or can be found here: http://www.ambusmartman.com/sup-documents.html 
	STEP 3: Start SmartMan


Simply click on the Icon  [image: image1.jpg]


..
	Detail on Installing the SmartMan USB Drivers


Under Vista, XP and Win 7 all USB connected devices must load the correct drivers. 

When you unzip the SmartMan software, it created a directory which contains the proper driver. You must tell the operating system to use these drivers.

The Procedure

Plug the USB cable in the manikin and into the computer.

You will see that Windows will detect a new device.

Windows will always ask if you "want to: 




"Search automatically for updated software" 




"Browse my computer for driver software" 
Always choose the "Browse my computer ............. option
The drivers are in one of two places depending on which operating system you have and whether it is 32 bit or 64 bit. Follow the onscreen instructions to point to one of the following locations:

C:\Program Files\SmartManv5.1\SmartManDriver

OR

C:\Program Files (x86)\SmartManv5.1\SmartManDriver

The operating system will take up to 2 minutes to copy and install the driver. 

The following Details are covered below
1. Installing on a Vista Operating system
2. Installing on an XP Operating System
3. Installing on a Win 7 Operating System
4. 4. How to install from the Device Manager


	1. Installing On a VISTA Operating System


When you first plug in the USB cable Vista will attempt to install a new device. When you see  the following message select the first option “Locate and install driver software”.

[image: image2.jpg]‘Windows needs to install driver software for your VM167
USB Interface Board

® Locate and install driver software (recommended)

Windows will guide you through the process of installing driver software
for your device.

2 Ask me again later
Windows il ssk agsi the next time you plug in your device or g on

® Don't show this message again for this device
Your device il not function untilyou instaldrver software.





If you see the following message, select “I don’t have the disc. Show me other options”. 

[image: image3.png]Insert the disc that came with your VM167 USB Interface Board

I you have the disc that came with your device, insert it now. Windows will automatically
search the disc for driver software.





This will bring up the following screen. Select the second option “Browse my computer for driver software”

[image: image4.png]‘Windows couldn't find driver software for your device

* Check for a solution

Windows will check to see if there are steps you can take to get your device
working.

2 Browse my computer for driver software (advanced)
Locte and nstll driver software manusly.





You will be given the chance to Browse for the driver. Click Browse and go to the following directory

C:\Program Files\SmartManv5.1\SmartManDriver

OR

C:\Program Files (x86)\SmartManv5.1\SmartManDriver

[image: image5.jpg]Browse for driver software on your computer

Search for driver software in this location:

Include subfolders





If you see the following security message click, “Install this driver software anyway.”

[image: image6.png]the publisher of this driv

* Don'tinstall this driver software

You should check your manufacturer's website for updated driver software
foryour device.

Install this driver software anyway

Onlyinstall driver software obtained from your manufacturer's website or
disc. Unsigned software from other sources may harm your computer o steal
information.

(V) Seedetss





The operating system will take up to 2 minutes to copy and install the driver. 

It will tell you that the device has been installed correctly.


	
2. Installing on an XP Operating System


When you plug in the USB cable you will be prompted by Windows with the ‘Found New hardware Wizard’.
[image: image7.jpg]tware Update Wizard,

Welcome to the Hardware Update
PNl wizard
¥

‘Windows wil search for current and updated software by
looking on your computer, on the hardware installtion CD. or on
the Windaws Update Web site [wih your permission.

ad ou privacy polt

Can Windows connect to Windows Update to search for
soltware?

O Yes, this time only
‘es, now and every time | connect a device

Ciick Next to cantinue.





Select ‘No, not this time’ and click ‘Next’ to continue

[image: image8.jpg]fware Update Wizard

This wizard helps you nstal software for

VM167 USE Interface Board

*)._If your hardware came with an installation CD
2 or floppy disk, insert it now.

What do you want the wizard to do?

Install the softuare automaticall (Recommended)

Ciick Next to continue.





Select ‘Install from a list or specific location (Advanced)’ Click ’Next’ to continue.

You will be asked for the location of your driver files. 
Select ‘Include this location in the search’ and browse to the following location
C:\Program Files\SmartManv5.1\SmartManDriver

OR

C:\Program Files (x86)\SmartManv5.1\SmartManDriver

[image: image9.jpg]Hardware Update Wizard

Please choose your search and installation options. E§

Search forthe best diiver in these locations.

Use the check boses below to lmit or expand the default ssarch, which includes local
paths and removable media. The best diver found will be installed.

] Search removable media (flopey. CD-ROM...)

Include this location in the search:

Crognan s GBoYSmarthan v ASmartiaiver v

O Don't search. | will chooss the driver to instal

Choose this opton to select the device diver from a fst. Windows does not guaranee that
the diiver you choose willbe the best match for yous hardware.





Click ‘Next’ to continue.
If you receive a warning, click ‘Continue Anyway’. And then click ‘Finish’. 
The operating system will take up to 2 minutes to copy and install the driver. 

It will tell you that the device has been installed correctly.



	3. Installing on a Windows 7 (32/64 bit) Home, Pro, Ultimate, Enterprise Operating System


Plug the USB cable into the manikin and into the computer.

When the program asks to install the device, select ‘Browse my computer for the driver software’ ’locate and install driver software manually.’ This is the second option
[image: image10.jpg]NS

| Update Driver Software - VM167 USB Interface Board

How do you want to search for driver software?

+ Search automatically for updated driver software
Windows will search your computer and the Internet for the latest driver software
for your device, unless you've disabled this feature in your device installation |
setings. |

>

Locate and instal driver software manually.

[ Cancel





Locate the driver software here 
C:\Program Files\SmartManv5.1\SmartManDriver

OR

C:\Program Files (x86)\SmartManv5.1\SmartManDriver

[image: image11.jpg]Browse for driver software on your computer

Search for driver software in this location:

‘C:iProgram Files (x86)\SmartMan v5.1\SmartManDriver

[Z]Include subfolders

4 Letme pick from a list of device drivers on my computer
This fist will show installed driver software compatible with the device, and all driver
software in the same category as the device.





Click ‘Next’ to continue.
When you receive the following warning, click ‘Install this software anyway’. 

[image: image12.jpg]2! Windows Security

9 Don'tinstall this driver software
You should check your manufacturer's website for updated driver software
for your device.

PY

only install driver software obtained from your manufacturer's website or

disc. Unsigned software from other sources may harm your computer or stea

) See details





Installation of the drivers should now be complete.

[image: image13.jpg]Windows has successfully updated your driver software

Windows has finished installing the driver software for this device:

VMI67 USB Interface Card

Ed





The operating system will take up to 2 minutes to copy and install the driver. 

It will tell you that the device has been installed correctly.



	4. Installing/Verifying Under Device Manager


All devices installed on your operating system will show in the Device Manager. When the USB is plugged in, it should show under “Customer USB Devices”.

Getting To Device manager
Option 1: Through the Control Panel

 Click on 'START' then select 'Control Panel'
[image: image14.jpg]Pictures
Music

Downloads

Games

Favortes
—

Control Panel
Devices and Pintes
Help and Support

Run.




Option 2: From The start Button

At the start Button, type “Device Manager”.

[image: image15.jpg]Programs (1)

Control Panel (3),

& Device Manager
& View devices and printers
) Update device drivers




[image: image16.jpg]5 seemore resuts

Device Manager x| [shutdown [ » |

[l





After you type it, it will appear where the red arrow is in the above picture. Click on “Device Manager” when it appears.
What You Will See In Device Manager
SmartMan should appear as a Customer US Device, such as in the following Win 7 example. If it installed correctly it will appear as in the following table.

[image: image17.jpg]g Owner-PC

@ Batteres

Computer
Custom USE Devicer
§ UMI6T USE Interface Card
SesTeae
. B, Diplay sdspters

- L DVD/CD-ROM drives
- g IDEATA/ATAR! contrllers.





If it is not there, there is a problem and you will see something like the following with the yellow exclamation marked icon.
[image: image18.jpg]4-[Ip Other devices
[l VMI67 USB Interface Board



.

What to do in Device Manager
From device manager you are able to 

a) Install the SmartMan USB drivers from scratch, or

b) Update the SmartMan USB drivers, or

c) Delete the driver and re-install it again if required.

Option 1: Update Driver Software
	[image: image19.png]48 Owner-PC
9 Botteries
48 Computer
b Disk drives
Display adapters
» <& DVD/CD-ROM drives
b -C IDE ATA/ATAPI controllers
25 Imaging devices
> -4) Intel(R) Centrino(R) WiMAX adapters
b Keyboards
I Mice and other pointing devices
> B8 Monitors
> 4 Netwo

T Other devices
VMI67 USB Interface Board

»& Sound, video and game controllers
b 188 System devices
- § UniversalSerial Bus controllers




	The device you are looking for is the 

VM167 USB Interface Board.

It should be listed under Custom USB Devices, but if there is a problem it will likely show under Other Devices with the yellow exclamation icon.

[image: image20.jpg]4-[Ip Other devices
[l VMI67 USB Interface Board




The yellow triangle with the exclamation mark means that it has not been installed. Either it was never installed or there was an error during installation


Often the operating system will recognize the device but the drivers are not properly linked to it. If there is a yellow triangle with an exclamation mark, either the drivers were not installed or there was an error during installation.

Select VM167 USB Interface Board.

Right click and in the window select ‘Update Driver Software’

Select to browse for the location of the SmartMan drivers. This will take you back to the beginning of the installation instructions listed above. 

If that option to ‘Update Driver Software’ is not there on the menu, you most likely have just plugged the USB cable into your computer. Wait for about 5 minutes and see if it has finished or select the next option.
Another Way in Device Manager
When in the Device Manager, another option is to go to Actions in the top menu bar and install the driver from scratch. In order to have any option under Actions, you must select (click once) on 
· Click on “Universal Serial Bus controllers”
· Click on Action
· Select “Add legacy hardware”
[image: image21.jpg]Add Hardware

Welcome to the Add Hardware Wizard

This wizard helps you installdriver software to support older
devices that do not support Plug-and-Play and which are not
automatically recognized by Windows.

You should only use this wizard if you are an advanced user or
you have been directed here by technical support,

L If your hardware came with an installation CD,

recommended that you click Cancel to close this wizard
and use the manufacturer’s CD to install this hardware.

To continue, click Next

( Net> D[ concel





 Click Next
[image: image22.jpg]‘Add Hardware

The wizard can help you install other hardware:

“The wizard can search for other hardware and automatically install it for you. Or, if you
know exactly which hardware model you want to instal, you can select t from a lst.

What do you want the wizard to do?

Search for and install the hardware automatically (Recommended)

T e )





Select to “Install the hardware manually”.

[image: image23.jpg]‘Add Hardware

From the list below. select the type of hardware you are installing

F you do not see the hardware category you want, click Show All Devices.

Common hardware types:

Custom USE Devices

.
GIDE ATA/ATAPI controlers
EEEE 1284.4 compatiole printer

EEEE 12844 devices

§ EEE139¢ Bus hostcontrllrs

2 imaging devices

F nfored devices -

Coa o)





Click on “Custom USB Devices”. Then click next.

[image: image24.jpg]Add Hardware

Select the device driver you want to install for this hardware.

Select the manufacturer and model of your harcware device and then click Next. I you have a
P diskthat contains the drver you want to instal, cick Have Disk.

Model
VMI67 USB Interface Card

/i, This driver is not digitally signed!
Tell me why driver signing i important





If SmartMan is plugged in, it should find the VM167 USB Interface Card.

Click on Have Disk.

[image: image25.jpg]Install From Disk

[ Insetthe manfacturers insalation dsk. and hen
2 make surethat the conect dive s selected below.

Copy manufacturer’s s from
y





Click on browse.

Then point the drivers to 

C:\Program Files\SmartManv5.1\SmartManDriver

OR

C:\Program Files (x86)\SmartManv5.1\SmartManDriver

	STEP 3: Start SmartMan


Simply click on the Icon  [image: image26.jpg]


..

2.   In Control Panel:


If the display is in Large/Small icons, select 'Device Manager'


If the display is in 'View By Category':


Select 'Hardware and Sound',


then 'Devices and Printers',


then 'Device manager'.








SmartMan User Manual – 1. Start 0. Install SmartMan                                        (v5.1 Nov 2011)    Page 13

